/*-----------------WEEK 2 TASK 1------------------*/

#include "stm32f4xx.h"

int main(void){

// ENABLE CLOCK TO GPIOA AND GPIOC (needed for PA5 internal LED and

PC pins)

RCC->AHB1ENR |= RCC_AHB1ENR_GPIOAEN | RCC_AHB1ENR_GPIOCEN;

// SET PC5, PC6, PC8, PC9 AS OUTPUTS (external LEDs)

GPIOC->MODER |= GPIO_MODER_MODER5_0 | GPIO_MODER_MODER6_0 |

GPIO_MODER_MODER8_0 | GPIO_MODER_MODER9_0 ;

// SET PA5 AS OUTPUT (internal LED)

GPIOA->MODER = GPIO_MODER_MODER5_0;

// SET PC13 AS INPUT (USER BUTTON)

GPIOC->MODER &= ~GPIO_MODER_MODER13_Msk;

// MAIN LOOP     while(1) {

// CHECK IF USER BUTTON IS CLICKED (PC13 reads LOW when pressed)         if(!(GPIOC->IDR & GPIO_IDR_ID13) == 0){

// Turn ON all external LEDs

GPIOC->BSRR |= GPIO_BSRR_BS5 | GPIO_BSRR_BS6 |

GPIO_BSRR_BS8 | GPIO_BSRR_BS9;

// Turn OFF internal LED (PA5)

GPIOA->BSRR |= GPIO_BSRR_BR5;

}         else {

// Turn OFF external LEDs

GPIOC->BSRR |= GPIO_BSRR_BR5 | GPIO_BSRR_BR6 |

GPIO_BSRR_BR8 | GPIO_BSRR_BR9;

// Turn ON internal LED

GPIOA->BSRR |= GPIO_BSRR_BS5;

}

}

}

/*---------------WEEK 2 TASK 2--------------------*/

#include "stm32f4xx.h"

// SIMPLE DELAY FUNCTION void delay_ms(uint32_t ms) {     volatile uint32_t i, j;     for (i = 0; i < ms; i++) {         for (j = 0; j < 1000; j++);

// Crude software delay (depends on CPU speed)

}

}  int main(void){     // ENABLE CLOCK TO GPIOA AND GPIOC

RCC->AHB1ENR |= RCC_AHB1ENR_GPIOAEN | RCC_AHB1ENR_GPIOCEN;

// SET PC5, PC6, PC8, PC9 AS OUTPUTS (LEDs)

GPIOC->MODER |= GPIO_MODER_MODER5_0 | GPIO_MODER_MODER6_0 |

GPIO_MODER_MODER8_0 | GPIO_MODER_MODER9_0 ;

while(1) {

// CHECK IF BUTTON ON PC10 IS *NOT PRESSED*         // (pull-down → reads 0 normally)         if(!(GPIOC->IDR & GPIO_IDR_ID10) == 1){

// LEFT → RIGHT ORDER: PC5 → PC6 → PC8 → PC9

GPIOC->BSRR |= GPIO_BSRR_BS5;    // Turn ON PC5             delay_ms(100);

GPIOC->BSRR |= GPIO_BSRR_BR5;    // Turn OFF PC5

GPIOC->BSRR |= GPIO_BSRR_BS6;    // Turn ON PC6             delay_ms(200);

GPIOC->BSRR |= GPIO_BSRR_BR6;    // Turn OFF PC6

GPIOC->BSRR |= GPIO_BSRR_BS8;    // Turn ON PC8             delay_ms(300);

GPIOC->BSRR |= GPIO_BSRR_BR8;    // Turn OFF PC8

GPIOC->BSRR |= GPIO_BSRR_BS9;    // Turn ON PC9             delay_ms(400);

GPIOC->BSRR |= GPIO_BSRR_BR9;    // Turn OFF PC9

}         else {

// BUTTON PRESSED → REVERSE ORDER (RIGHT → LEFT)

// PC9 → PC8 → PC6 → PC5

GPIOC->BSRR |= GPIO_BSRR_BS9;    // Turn ON PC9             delay_ms(100);

GPIOC->BSRR |= GPIO_BSRR_BR9;

GPIOC->BSRR |= GPIO_BSRR_BS8;    // Turn ON PC8

delay_ms(200);

GPIOC->BSRR |= GPIO_BSRR_BR8;

GPIOC->BSRR |= GPIO_BSRR_BS6;    // Turn ON PC6

delay_ms(300);

GPIOC->BSRR |= GPIO_BSRR_BR6;

GPIOC->BSRR |= GPIO_BSRR_BS5;    // Turn ON PC5

delay_ms(400);

GPIOC->BSRR |= GPIO_BSRR_BR5;

}

delay_ms(200); // Delay between cycles

}

}

/*--------------------WEEK 3-----------------------*/

int myLed = 6;          // LED connected to digital pin 6 int input = 0;          // Variable to store serial input

void setup() {

pinMode(myLed,OUTPUT);      // Configure pin 6 as an output

Serial.begin(115200);       // Start serial communication at 115200 baud

while(!Serial);             // Wait until serial port is ready (for boards like Leonardo)

Serial.println("Serial is ready!");   // Print startup message

}

void loop() {

// Check if data is available in the Serial buffer     if(Serial.available() > 0){

input = Serial.read();   // Read a single character from the serial port

// Check if the input is a letter (uppercase or lowercase)         if(input >= 'a' && input <= 'z' || input >= 'A' && input <= 'Z'){

Serial.println("Letter Pressed ");   // Print that a letter was received

digitalWrite(myLed,HIGH);            // Turn ON the LED

}          else{

Serial.println("Number Pressed ");   // Print that a number/other char was received

digitalWrite(myLed,LOW);             // Turn OFF the LED         }

Serial.flush();   // Wait for all outgoing serial data to be sent     }

delay(5);  // Small delay to avoid reading too fast

}

/*--------------------------WEEK 4 TASK 1 ---------------------------------------*/

int pin = A0;   // Using analog input A0

void setup() {

Serial.begin(9600);   // Start serial communication at 9600 baud   while(!Serial);       // Wait for Serial monitor (important for some boards)

}  void loop() {

uint16_t rawADC = analogRead(pin);   // Read 10-bit ADC value (0–1023)

// Convert ADC value to voltage (3.3V reference, 10-bit resolution)   float voltage = 3.3 * rawADC / 1023.0;

// Print raw ADC value

Serial.print("ADC Value = ");

Serial.print(rawADC);

Serial.println(".");

// Print corresponding voltage

Serial.print("Voltage = ");

Serial.println(voltage);

Serial.println(" V.");

delay(1000);   // 1-second delay

}

/*----------------------------WEEK 4 TASK 2-------------------------------*/

int tmp36 = A0;   // TMP36 sensor connected to analog pin A0

void setup() {

Serial.begin(9600);   // Start serial communication   while(!Serial);       // Wait for Serial monitor to open

analogReadResolution(12);

// Set ADC resolution to 12 bits (0–4095)

// Needed for more precision on boards that support it

}  void loop() {

uint16_t rawADC = analogRead(tmp36);  // Read 12-bit ADC value (0–4095)

// Convert ADC value to voltage using 3.3V reference and 12-bit scale   float voltage = 3.3 * rawADC / 4095.0;

// TMP36 formula: Temperature (°C) = (Vout - 0.5V) × 100   float temperature = (voltage - 0.5) * 100;

// Print calculated temperature

Serial.print("Room Temperature = ");

Serial.print(temperature);

Serial.println(" Degree Celcius.");

delay(1000);  // 1-second delay

}

/*------------------------WEEK 5  Uart_driver c -------------------------

-*/

#include "uart_driver.h"

#include "stm32f4xx.h"

#include <stdbool.h>

// Timeout value (in ms)

#define UART_TIMEOUT_MS 1000

// Global millisecond counter

static volatile uint32_t sysTick_ms = 0;

// Initialise SysTick for 1ms interrupts static void SysTick_Init(void) {

// Configure SysTick to interrupt every 1ms

SysTick->LOAD = (16000000 / 1000) - 1; // 16 MHz / 1000 = 16000 - 1

SysTick->VAL = 0; // Clear current value

SysTick->CTRL = SysTick_CTRL_CLKSOURCE_Msk |

SysTick_CTRL_TICKINT_Msk |

SysTick_CTRL_ENABLE_Msk; // Enable SysTick, CPU clock, and interrupt }

// SysTick Handler (increments ms counter) void SysTick_Handler(void) { sysTick_ms++;

}

// Return system tick in ms inline uint32_t GetTick(void) { return sysTick_ms;

}

// ====================== UART DRIVER ======================

// GPIO Configuration for USART2

// PA2 - TX (AF7)

// PA3 - RX (AF7)

int __io_putchar(int ch){ UART2_TransmitByte((uint8_t)ch); return ch;

}

UART_Status_t UART2_Init(UART_Config_t* config) {

// Enable SysTick if not already running

SysTick_Init();

// Enable clocks

RCC->AHB1ENR |= RCC_AHB1ENR_GPIOAEN; // GPIOA clock

RCC->APB1ENR |= RCC_APB1ENR_USART2EN; // USART2 clock

// Configure GPIO pins PA2 (TX) and PA3 (RX)

GPIOA->MODER &= ~(GPIO_MODER_MODER2 | GPIO_MODER_MODER3);

GPIOA->MODER |= (GPIO_MODER_MODER2_1 | GPIO_MODER_MODER3_1);

GPIOA->OSPEEDR |= (GPIO_OSPEEDER_OSPEEDR2 | GPIO_OSPEEDER_OSPEEDR3);

GPIOA->PUPDR &= ~(GPIO_PUPDR_PUPDR2 | GPIO_PUPDR_PUPDR3);

GPIOA->PUPDR |= GPIO_PUPDR_PUPDR3_0; // Pull-up on RX

GPIOA->AFR[0] &= ~(GPIO_AFRL_AFRL2 | GPIO_AFRL_AFRL3);

GPIOA->AFR[0] |= (7 << GPIO_AFRL_AFSEL2_Pos) | (7 <<

GPIO_AFRL_AFSEL3_Pos);

// Reset USART2

USART2->CR1 = 0;

USART2->CR2 = 0;

USART2->CR3 = 0;

// Configure USART2

USART2->CR1 |= config->word_length | config->parity;

USART2->CR2 |= config->stop_bits;

// Set baud rate

if (UART2_SetBaudRate(config->baudrate) != UART_OK) { return UART_ERROR;

}

// Enable USART2, transmitter and receiver

USART2->CR1 |= USART_CR1_UE | USART_CR1_TE | USART_CR1_RE;

return UART_OK;

}

UART_Status_t UART2_DeInit(void) {

USART2->CR1 &= ~USART_CR1_UE;

RCC->APB1ENR &= ~RCC_APB1ENR_USART2EN;

GPIOA->MODER &= ~(GPIO_MODER_MODER2 | GPIO_MODER_MODER3); return UART_OK;

}

UART_Status_t UART2_SetBaudRate(uint32_t baudrate) { uint32_t pclk = 16000000; // APB1 at 16 MHz default uint32_t tmp = (pclk + baudrate/2) / baudrate;

if (tmp < 16 || tmp > 0xFFFF) { return UART_ERROR;

}

USART2->BRR = tmp; return UART_OK;

}

UART_Status_t UART2_Transmit(uint8_t* data, uint16_t size, uint32_t timeout) {

uint32_t start_tick = GetTick();

for (uint16_t i = 0; i < size; i++) { while (!(USART2->SR & USART_SR_TXE)) { if ((GetTick() - start_tick) > timeout) { return UART_TIMEOUT;

}

}

USART2->DR = data[i];

}

while (!(USART2->SR & USART_SR_TC)) { if ((GetTick() - start_tick) > timeout) { return UART_TIMEOUT;

}

}

return UART_OK;

}

UART_Status_t UART2_Receive(uint8_t* data, uint16_t size, uint32_t timeout) {

uint32_t start_tick = GetTick();

for (uint16_t i = 0; i < size; i++) { while (!(USART2->SR & USART_SR_RXNE)) { if ((GetTick() - start_tick) > timeout) { return UART_TIMEOUT;

} }

data[i] = (uint8_t)(USART2->DR & 0xFF);

}

return UART_OK;

}

UART_Status_t UART2_TransmitByte(uint8_t data) { uint32_t start_tick = GetTick();

while (!(USART2->SR & USART_SR_TXE)) { if ((GetTick() - start_tick) > UART_TIMEOUT_MS) { return UART_TIMEOUT;

}

}

USART2->DR = data; return UART_OK;

}

uint8_t UART2_ReceiveByte(void) { while (!(USART2->SR & USART_SR_RXNE)); return (uint8_t)(USART2->DR & 0xFF);

}

bool UART2_IsDataAvailable(void) { return (USART2->SR & USART_SR_RXNE) != 0;

}  bool UART2_IsTransmitComplete(void) { return (USART2->SR & USART_SR_TC) != 0;

}

/*--------------UART_driver. H-----------*/

#ifndef UART_DRIVER_H_

#define UART_DRIVER_H_

#include "stm32f4xx.h"

#include <stdint.h>

#include <stdbool.h>

#include <stdio.h>

// ================= UART CONFIGURATION ================= typedef struct { uint32_t baudrate;

uint32_t word_length; // USART_CR1_M (0 = 8 bits, 1 = 9 bits) uint32_t stop_bits; // USART_CR2_STOP (00 = 1 bit, 10 = 2 bits) uint32_t parity; // USART_CR1_PCE and USART_CR1_PS

} UART_Config_t;

// Predefined configurations

#define UART_WORDLENGTH_8B 0x00000000U

#define UART_WORDLENGTH_9B USART_CR1_M

#define UART_STOPBITS_1 0x00000000U

#define UART_STOPBITS_2 USART_CR2_STOP_1

#define UART_PARITY_NONE 0x00000000U

#define UART_PARITY_EVEN USART_CR1_PCE

#define UART_PARITY_ODD (USART_CR1_PCE | USART_CR1_PS)

// ================= UART STATUS ================= typedef enum { UART_OK = 0,

UART_ERROR,

UART_BUSY,

UART_TIMEOUT

}

UART_Status_t;

// ================= UART DRIVER =================

UART_Status_t UART2_Init(UART_Config_t* config);

UART_Status_t UART2_DeInit(void);

UART_Status_t UART2_SetBaudRate(uint32_t baudrate);

UART_Status_t UART2_Transmit(uint8_t* data, uint16_t size, uint32_t timeout);

UART_Status_t UART2_Receive(uint8_t* data, uint16_t size, uint32_t timeout_ms);

UART_Status_t UART2_TransmitByte(uint8_t data); uint8_t UART2_ReceiveByte(void); bool UART2_IsDataAvailable(void); bool UART2_IsTransmitComplete(void);

// ================= SYSTICK API =================

// SysTick is initialised inside UART2_Init() // but these are provided for external use if needed.

void SysTick_Handler(void); // ISR (increments millisecond counter) uint32_t GetTick(void); // Return current millisecond tick

#endif /* UART_DRIVER_H_ */

/*---------------------WEEK 6--------------*/

#include "stm32f4xx.h"

volatile uint32_t tim3_overflow = 0;  volatile uint32_t last_capture = 0;  volatile uint32_t period_ticks = 0;  volatile float period_ms = 0;

void inputCapture_TIMCH4(void){  RCC->AHB1ENR |= RCC_AHB1ENR_GPIOCEN;

RCC->APB1ENR |= RCC_AHB1ENR_TIM3EN;

GPIOC->MODER &= ~GPIO_MODER_MODER9;

GPIOC->MODER |= GPIO_MODER_MODER9_1;

GPIO->AFR[1] &= ~GPIO_AFRH_AFSEL9;

GPIO->AFR[1] |= 4 << GPIO_AFRH_AFSEL9_Pos;

TIM3->PSC = 1-1;

TIM3->ARR = 0xFFFF; // 4.1 ms

TIM3->CCMR2 |= TIM_CCMR2_CC4S_0;

TIM3->CCER &= ~TIM_CCER_CC4P;

TIM3->CCER &= ~TIM_CCER_CC4NP;

TIM3->CCER |= TIM_CCER_CC4E;

TIM3->DIER |= TIM_DIER_UIE | TIM_DIER_CC4IE;

TIM3->EGR |= TIM_EGR_UG;

TIM3->CR1 |= TIM_CR1_CEN;

NVIC_EnableIRQ(TIM3_IRQn);

}

int main(void){

while(1){};

}    void TIM3_IRQHandler(void){  if(TIM3->SR & TIM_SR_UIF){  tim3_overflow++;  TIM3->SR &= ~TIM_SR_UIF;

}    if(TIM->SR & TIM_SR_CC4IF){  TIM3_IC_callback();

TIM3->SR &= ~TIM_SR__CC4IF;

}

}    void TIM3_IC_callback(void){  uint32_t capture = TIM3->CCR4;

uint32_t timestamp = (tim3_overflow << 16) | capture;  period_ticks = timestamp - last_capture;  last_capture = timestamp;  period_ms =1000/period_ticks/16000000;

}

/*-----------------WEEK 7 --------------------*/

#include "stm32f4xx.h"

volatile uint32_t tim3_overflow = 0;  volatile uint32_t last_capture = 0;  volatile uint32_t period_ticks = 0;  volatile float period_ms = 0;

void inputCapture_TIMCH4(void){  RCC->AHB1ENR |= RCC_AHB1ENR_GPIOCEN;

RCC->APB1ENR |= RCC_AHB1ENR_TIM3EN;

GPIOC->MODER &= ~GPIO_MODER_MODER9;

GPIOC->MODER |= GPIO_MODER_MODER9_1;

GPIO->AFR[1] &= ~GPIO_AFRH_AFSEL9;

GPIO->AFR[1] |= 4 << GPIO_AFRH_AFSEL9_Pos;

TIM3->PSC = 1-1;

TIM3->ARR = 0xFFFF; // 4.1 ms

TIM3->CCMR2 |= TIM_CCMR2_CC4S_0;

TIM3->CCER &= ~TIM_CCER_CC4P;

TIM3->CCER &= ~TIM_CCER_CC4NP;

TIM3->CCER |= TIM_CCER_CC4E;

TIM3->DIER |= TIM_DIER_UIE | TIM_DIER_CC4IE;

TIM3->EGR |= TIM_EGR_UG;

TIM3->CR1 |= TIM_CR1_CEN;

NVIC_EnableIRQ(TIM3_IRQn);

}

int main(void){

while(1){};

}    void TIM3_IRQHandler(void){  if(TIM3->SR & TIM_SR_UIF){  tim3_overflow++;  TIM3->SR &= ~TIM_SR_UIF;

}

if(TIM->SR & TIM_SR_CC4IF){  TIM3_IC_callback();

TIM3->SR &= ~TIM_SR__CC4IF;

}

}    void TIM3_IC_callback(void){  uint32_t capture = TIM3->CCR4;

uint32_t timestamp = (tim3_overflow << 16) | capture;  period_ticks = timestamp - last_capture;  last_capture = timestamp;

period_ms =1000/period_ticks/16000000;

}

WEEK 7:

Week 7 TASK1

Create a program using the I2C protocol to read the acceleration from the  ADXL345

MAIN.C

#include "stm32f4xx.h"

#include <stdio.h>

#include <stdint.h>

#include "adxl345.h"

int16_t x,y,z;  float xg, yg, zg;

extern uint8_t rec_data[6];

int main(void){

adxl_init();

while(1){

adxl_read_values(DATA_START_ADD);

x = ((rec_data[1] << 8) | rec_data[0]);  y = ((rec_data[3] << 8) | rec_data[2]);  z = ((rec_data[5] << 8) | rec_data[4]);

xg = x*0.0078;  yg = y*0.0078;  zg = z*0.0078;

}

}

I2c_drive.c

#include "i2c_driver.h"

/*

- Pinout for the Nucleo-F411RE

- PB8 ------ SCL

- PB9 ------ SDA

- I am using those for I2C1

*/

/*

- From 18.6.8 in the reference manual

- CCR = fclk/(2fscl)

- CCR = 16E6/(2*100E3)

- CCR = 80 or 0x50

*/

/*

- From 18.6.9 in the reference manual

- F = 16 MHz -> T = 62.5 ns

- F = 100 kHz -> T

- TRise = 1000 ns/ 62.5 ns +1

- TRise = 17

*/

| #define I2C_100kHz |  | 0x50 |
| --- | --- | --- |
| #define Sm_MAX_RISE_TIME | 17 |  |

void I2C1_init(void){

RCC->AHB1ENR |= RCC_AHB1ENR_GPIOBEN;

GPIOB->MODER &= (~GPIO_MODER_MODER8_Msk) | ~(GPIO_MODER_MODER9_Msk);

GPIOB->MODER |= GPIO_MODER_MODER8_1 | GPIO_MODER_MODER9_1;

GPIOB->AFR[1] |= (4U<<GPIO_AFRH_AFSEL8_Pos);

GPIOB->AFR[1] |= (4U<<GPIO_AFRH_AFSEL9_Pos);

GPIOB->OTYPER |= (GPIO_OTYPER_OT8 | GPIO_OTYPER_OT9);

GPIOB->PUPDR |= (GPIO_PUPDR_PUPD8_0 | GPIO_PUPDR_PUPD9_0);

RCC->APB1ENR |= RCC_APB1ENR_I2C1EN;

I2C1->CR1 |= I2C_CR1_SWRST;

I2C1->CR1 &= ~I2C_CR1_SWRST_Msk;

I2C1->CR2 |= (1U << 4);

I2C1->CCR = I2C_100kHz;

I2C1->TRISE = Sm_MAX_RISE_TIME;

I2C1->CR1 |= I2C_CR1_PE;

}

void I2C1_readByte(char saddr, char maddr, char* data){

volatile int tmp;

while(I2C1->SR2 & I2C_SR2_BUSY){}

I2C1->CR1 |= I2C_CR1_START;

while(!(I2C1->SR1 & I2C_SR1_SB)){}

I2C1->DR = (saddr << 1);

while(!(I2C1->SR1 & I2C_SR1_ADDR)){}

tmp = I2C1->SR2;

I2C1->DR = maddr;

while(!(I2C1->SR1 & I2C_SR1_TXE)){}

I2C1->CR1 |= I2C_CR1_START;

while(!(I2C1->SR1 & I2C_SR1_SB)){}

I2C1->DR = saddr << 1 | 1;

while(!(I2C1->SR1 & I2C_SR1_ADDR)){}

I2C1->CR1 &= ~I2C_CR1_ACK_Msk;

tmp = I2C1->SR2;

I2C1->CR1 |= I2C_CR1_STOP;

while(!(I2C1->SR1 & I2C_SR1_RXNE)){}

*data++ = I2C1->DR;

(void)tmp;

}

void I2C1_burstRead(char saddr, char maddr, int n, char* data){  volatile int tmp;

while(I2C1->SR2 & I2C_SR2_BUSY){}

I2C1->CR1 |= I2C_CR1_START;

while(!(I2C1->SR1 & I2C_SR1_SB)){}

I2C1->DR = (saddr << 1);

while(!(I2C1->SR1 & I2C_SR1_ADDR)){}

tmp = I2C1->SR2;

while(!(I2C1->SR1 & I2C_SR1_TXE)){}

I2C1->DR = maddr;

while(!(I2C1->SR1 & I2C_SR1_TXE)){}

I2C1->CR1 |= I2C_CR1_START;

while(!(I2C1->SR1 & I2C_SR1_SB)){}

I2C1->DR = saddr << 1 | 1;

while(!(I2C1->SR1 & I2C_SR1_ADDR)){}

tmp = I2C1->SR2;

I2C1->CR1 |= I2C_CR1_ACK;

while(n > 0U){

if(n == 1U){

I2C1->CR1 &= ~I2C_CR1_ACK;

I2C1->CR1 |= I2C_CR1_STOP;

while(!(I2C1->SR1 & I2C_SR1_RXNE)){}

*data++ = I2C1->DR;  break;  }  else{

while(!(I2C1->SR1 & I2C_SR1_RXNE)){}

*data++ = I2C1->DR;  n--;  }

}

(void)tmp;

}

void I2C1_burstWrite(char saddr, char maddr, int n, char* data){

volatile int tmp;

while(I2C1->SR2 & I2C_SR2_BUSY){}

I2C1->CR1 |= I2C_CR1_START;

while(!(I2C1->SR1 & I2C_SR1_SB)){}

I2C1->DR = (saddr << 1);

while(!(I2C1->SR1 & I2C_SR1_ADDR)){}

tmp = I2C1->SR2;

while(!(I2C1->SR1 & I2C_SR1_TXE)){}

I2C1->DR = maddr;

for(int i = 0; i < n; i++){  while(!(I2C1->SR1 & I2C_SR1_TXE)){}

I2C1->DR = *data++;

}

while(!(I2C1->SR1 & I2C_SR1_BTF)){}

I2C1->CR1 |= I2C_CR1_STOP;

(void)tmp;

}

Ic2_drive.h

#ifndef I2C_DRIVER_H_

#define I2C_DRIVER_H_

#include "stm32f4xx.h"

void I2C1_init(void);

void I2C1_readByte(char saddr, char maddr, char* data);  void I2C1_burstRead(char saddr, char maddr, int n, char* data);  void I2C1_burstWrite(char saddr, char maddr, int n, char* data);

#endif /* I2C_DRIVER_H_ */

adxl345.C

#include "adxl345.h"

char data;

uint8_t rec_data[6];

void adxl_read_address(uint8_t reg){

I2C1_readByte(DEVICE_ADDR, reg, &data);

}

void adxl_write(uint8_t reg, char value){

char data[1];  data[0] = value;

I2C1_burstWrite(DEVICE_ADDR, reg, 1, data);

}

void adxl_read_values(uint8_t reg){

I2C1_burstRead(DEVICE_ADDR, reg, 6, (char*)rec_data);

}

void adxl_init(void){

I2C1_init();

adxl_read_address(DEVID);

adxl_write(DATA_FORMAT, FOUR_G);

adxl_write(PWR_CTL, RESET);  adxl_write(PWR_CTL, SET_MEASURE_B);

}